# Software Requirements Specification (SRS)

Project Name: Sistema de Iluminación Inteligente

Version: 1.0

Date: 16-Febrero-2026

Author(s): Luis Antonio , Luis Alejandro, Juan Luis , Uriel Everardo

Responsibles Signs: Luis Antonio , Luis Alejandro, Juan Luis , Uriel Everardo

1. Introduction

1.1 Purpose

This document establishes the system requirements for the Smart Lighting System with Motion Detection. It defines the functional and non-functional requirements, constraints, and interfaces necessary for the development of an embedded system that automatically controls residential lighting based on human presence detection while respecting user manual preferences.1.2 Scope

The Smart Lighting System is an embedded device designed to solve the problem of energy waste and inconvenience in residential lighting control.

The system:

- Automatically detects human presence using PIR (Passive Infrared) sensors

- Controls lighting activation/deactivation based on motion detection

- Recognizes and respects manual user interventions

- Optimizes energy consumption in residential spaces

- Provides simple configuration interface for user preferences

The system does NOT include (out of scope for version 1.0):

- Internet connectivity or remote control

- Voice assistant integration

- Multi-room simultaneous control

- Advanced home automation features1.3 Definitions, Acronyms, and Abbreviations

PIR - Passive Infrared (motion sensor technology)

MCU - Microcontroller Unit

GPIO - General Purpose Input/Output

PWM - Pulse Width Modulation

SPDT - Single Pole Double Throw (relay type)

AC - Alternating Current

DC - Direct Current

EEPROM - Electrically Erasable Programmable Read-Only Memory

Timeout - Predefined time period before automatic action

Override - Manual intervention that cancels automatic behavior

MTBF - Mean Time Between Failures

I2C - Inter-Integrated Circuit (communication protocol)

UART - Universal Asynchronous Receiver-Transmitter

LED - Light Emitting Diode1.4 References

[1] HC-SR501 PIR Motion Sensor Datasheet, RevA, 2019

[2] ESP32 Technical Reference Manual, Espressif Systems, Version 4.6, 2023

[3] Arduino Nano Datasheet and Specifications, Arduino.cc

[4] NOM-001-SEDE-2012 - Instalaciones Eléctricas (Utilización), México

[5] IEEE 830-1998 - Standard for Software Requirements Specifications

[6] Songle SRD-05VDC-SL-C Relay Datasheet

[7] IEC 61010-1 - Safety requirements for electrical equipment1.5 Document Overview

This document is organized as follows:

Section 2 (Overall Description): Provides context about the system, including product perspective, main functions, user characteristics, constraints, and assumptions.

Section 3 (Specific Requirements): Details all functional requirements (FR), non-functional requirements (NFR), external interfaces, real-time constraints, and safety/regulatory compliance.

Section 4 (Verification and Validation): NOT INCLUDED IN THIS RELEASE - Will describe testing methodologies in future versions.

Section 5 (Traceability Matrix): NOT INCLUDED IN THIS RELEASE - Will provide requirement tracking in future versions.2. Overall Description2.1 Product Perspective

The Smart Lighting System is a standalone embedded device that integrates into existing residential electrical infrastructure without requiring major structural modifications.

System Context:

- Operates independently without external systems

- Interfaces with standard AC electrical installations (110-127V México)

- Compatible with existing light switches and fixtures

- Self-contained with onboard microcontroller, sensor, and relay

Main Components:

- Microcontroller: ESP32 or Arduino Nano (decision pending)

- Motion Sensor: HC-SR501 PIR sensor

- Actuator: SPDT 10A relay with optical isolation

- Power Supply: AC/DC converter 5V 1A

- User Interface: Status LED and configuration button2.2 Product Functions

Main functional capabilities:

1. Automatic Presence Detection

- Detects human motion within 3-7 meter range

- 110° detection angle

- Response time < 500ms

2. Intelligent Light Control

- Automatic light activation upon motion detection

- Automatic deactivation after configurable timeout (30s - 15min)

- Manual override detection and respect

3. User Configuration

- Adjustable motion sensor sensitivity

- Configurable timeout duration

- Manual/automatic mode switching

- Parameter persistence (EEPROM storage)

4. System Status Indication

- Visual feedback via LED indicator

- Green: Automatic mode active

- Red: Manual mode active

- Blinking: Configuration mode

5. Safety Features

- Overload protection

- Optical isolation between control and power circuits

- Fail-safe behavior (maintains last known state on MCU failure)2.3 User Characteristics

End Users:

- Technical Level: Basic (comparable to using standard light switches)

- Age Range: Adults and elderly

- Experience: No electronics knowledge required

- Expectations: "Set and forget" operation with intuitive manual control

- Language: Spanish (primary), English (documentation)

Installers:

- Technical Level: Basic electrical knowledge

- Certifications: Licensed electricians (preferred)

- Skills: Ability to read simple wiring diagrams

- Familiarity: NOM-001-SEDE electrical standards

Maintenance Personnel:

- Technical Level: Basic troubleshooting skills

- Skills Required: Interpret LED indicators, use physical configuration buttons

- No Programming: Configuration via physical interface only2.4 Constraints

Hardware Constraints:

- Operating Voltage: 110-127V AC ±10% (Mexican standard)

- Maximum Load: 500W (relay limitation)

- PIR Detection Range: 3-7 meters maximum

- Detection Angle: 110° (sensor hardware limit)

- MCU Memory: Limited flash/RAM (< 4MB flash, < 520KB RAM for ESP32)

Environmental Constraints:

- Operating Temperature: 0°C to 50°C

- Relative Humidity: < 80% non-condensing

- Indoor Use Only: Not weatherproof

Regulatory Constraints:

- Compliance: NOM-001-SEDE-2012 mandatory

- Safety Certification: Required before commercial deployment

- EMI/EMC: Must not interfere with other electronic devices

Design Constraints:

- Low Cost: Target BOM < $20 USD per unit

- Size: Must fit standard electrical box (10cm x 10cm x 5cm max)

- No Internet: Version 1.0 operates offline only

- Single Room: Controls one light fixture only2.5 Assumptions and Dependencies

Assumptions:

- User has functional and code-compliant electrical installation

- Installation location has direct line-of-sight to coverage area

- Light fixtures are compatible (LED, incandescent, fluorescent < 500W)

- Users will not attempt to modify internal circuitry

- Ambient temperature remains within operating range

- Power supply is stable (no frequent brownouts or surges)

Dependencies:

- Component Availability: HC-SR501 PIR sensors, ESP32/Arduino, relays

- Stable Power: Reliable 110-127V AC supply

- Proper Installation: Correct sensor orientation and positioning

- Initial Calibration: Sensor tuning for specific environment

- User Manual: Adequate documentation for installers and end-users3. Specific Requirements3.1 Functional RequirementsFR-01: The system shall detect human motion within a range of 3 to 7 meters using a PIR sensor with response time < 500ms and false positive rate < 5% under normal conditions.

FR-02: The system shall automatically activate the connected light fixture within 1 second of detecting motion when operating in automatic mode and the light is currently off.

FR-03: The system shall automatically deactivate the connected light after a configurable timeout period (default 5 minutes, range 30s-15min) when no motion is detected.

FR-04: The system shall detect manual light deactivation by the user and enter "manual override mode" for a configurable period (default 30 minutes), during which automatic control is suspended.

FR-05: The system shall allow users to switch between automatic and manual modes via a physical button (double-press for manual mode, triple-press for automatic mode).

FR-06: The system shall provide configurable parameters including: sensor sensitivity, timeout duration, and override period, accessible via physical configuration interface.

FR-07: The system shall store all user configuration parameters in non-volatile memory (EEPROM) to persist settings after power loss.

FR-08: The system shall indicate its current operational state via LED: solid green (automatic mode active), solid red (manual mode), blinking (configuration mode).

FR-09: The system shall provide optional pre-warning (10-second light flashing) before automatic deactivation when enabled in configuration.

FR-10: The system shall reset to automatic mode after the manual override timeout period expires, unless user has set permanent manual mode.3.2 Non-Functional RequirementsNFR-01: Performance

- Motion detection to light activation latency: < 1 second (95th percentile)

- Relay switching time: < 100ms

- Configuration parameter update: < 500ms

- System boot time: < 3 seconds after power-on

NFR-02: Power Consumption

- Standby mode (idle, light off): < 3W

- Active mode (monitoring, light off): < 5W

- Configuration mode: < 6W

- Total system consumption excludes connected light fixture load

NFR-03: Reliability / Availability

- MTBF: > 8,000 hours continuous operation

- Failure rate: < 1% in first year of operation

- Automatic recovery from temporary power loss

- Graceful degradation: On sensor failure, maintain last known light state

NFR-04: Security

- Physical tampering detection (optional): Alert via LED if enclosure is opened

- Configuration lock: Prevent accidental parameter changes (button combination required)

- No wireless interfaces: Eliminates remote attack surface (v1.0)

NFR-05: Maintainability

- Modular design: PIR sensor, relay, and MCU are replaceable components

- Diagnostic mode: Accessible via button sequence, displays system health via LED blink patterns

- Firmware update capability: Reserved UART/USB port for future updates (ESP32)

- Clear labeling: All connectors and terminals labeled on PCB

NFR-06: Usability

- Initial setup time: < 10 minutes for trained installer

- User configuration: < 5 minutes without reading manual (intuitive button interface)

- LED feedback: Immediate visual confirmation of all user actions

- Fail-safe default settings: System operates safely out-of-box without configuration

NFR-07: Compatibility

- Input voltage range: 110-127V AC ±10% (98-140V AC)

- Frequency: 60Hz ±2Hz

- Load compatibility: Resistive (incandescent), inductive (fluorescent with ballast), capacitive (LED drivers)

- Maximum load: 500W resistive, 300W inductive, 200W LED

NFR-08: Environmental

- Operating temperature: 0°C to 50°C

- Storage temperature: -20°C to 70°C

- Relative humidity: 10% to 80% non-condensing

- Altitude: Up to 2000m above sea level3.3 External Interface Requirements3.3.1 User Interfaces

Physical Interfaces:

1. Status LED (RGB or dual-color):

- Solid Green: Automatic mode active, monitoring

- Solid Red: Manual mode active

- Blinking Green (fast): Motion detected

- Blinking Red (slow): Configuration mode

- Blinking Yellow: Error/warning state

2. Configuration Button:

- Single press: No action (prevent accidental activation)

- Long press (3s): Enter configuration mode

- Double press (< 500ms apart): Switch to manual mode

- Triple press (< 500ms apart): Switch to automatic mode

- In config mode: Short press cycles through parameters

- In config mode: Long press (3s) saves and exits

3. Optional: Small OLED display (future enhancement)

- Show current mode, timeout remaining, sensor status3.3.2 Hardware Interfaces

Sensors:

1. PIR Motion Sensor (HC-SR501):

- Interface: Digital GPIO (3.3V logic level)

- Pin: GPIO_SENSOR_IN (e.g., GPIO 4 on ESP32)

- Signal: HIGH when motion detected, LOW when idle

- Power: 5V DC, < 65mA

Actuators:

1. Relay Module (SPDT 10A):

- Interface: Digital GPIO (3.3V/5V logic)

- Pin: GPIO_RELAY_CTRL (e.g., GPIO 5 on ESP32)

- Signal: HIGH = relay closed (light ON), LOW = relay open (light OFF)

- Power: 5V DC, < 70mA coil current

- Isolation: Optical isolator between MCU and relay coil

Communication Buses (Future Expansion):

1. I2C (reserved):

- SDA/SCL pins for optional OLED display

- Clock speed: 100kHz (standard mode)

2. UART (programming/debug):

- Baud rate: 115200 bps

- USB-to-UART adapter for firmware updates

- TX/RX pins accessible via header

Power Supply:

1. AC/DC Converter:

- Input: 110-127V AC, 60Hz

- Output: 5V DC, 1A minimum

- Protection: Overvoltage, overcurrent, short-circuit

Electrical Connections:

1. AC Input: 2-wire connection (Line, Neutral)

2. AC Output (Load): 2-wire connection (Switched Line, Neutral)

3. Ground: Safety ground connection to metal enclosure3.3.3 Software Interfaces

Internal Software Modules:

1. Sensor Driver:

- Reads PIR GPIO state

- Implements debouncing (50ms)

- Generates motion events

2. Relay Controller:

- Controls GPIO state for relay

- Implements transition delays (prevent rapid switching)

3. State Machine:

- Manages operating modes (auto, manual, config)

- Handles timeout logic

- Processes user button inputs

4. Configuration Manager:

- Reads/writes EEPROM

- Validates parameter ranges

- Provides default values

5. LED Controller:

- Generates LED patterns (solid, blink, fade)

- Indicates system state

External Software Interfaces (v1.0):

- None (standalone operation)

Future Interfaces (v2.0+):

- MQTT protocol for IoT integration

- HTTP REST API for web configuration

- Serial command interface for debugging3.4 Real-Time Requirements

Critical Timing Constraints:

RT-01: Motion Detection Latency

- Deadline: 500ms from motion event to MCU detection

- Priority: High

- Consequence of miss: User perceives lag, poor experience

RT-02: Light Activation Response

- Deadline: 1000ms from motion detection to light ON

- Priority: Critical

- Consequence of miss: User frustration, system appears broken

RT-03: Relay Switching Time

- Deadline: 100ms from MCU command to relay state change

- Priority: High

- Consequence of miss: Accumulates to total response latency

RT-04: Button Debouncing

- Window: 50ms debounce period

- Priority: Medium

- Consequence of miss: False triggers, unreliable user input

RT-05: Timeout Accuracy

- Tolerance: ±2 seconds for timeout periods

- Priority: Low

- Consequence of miss: Minor user inconvenience

RT-06: Configuration Save

- Deadline: 500ms from user confirmation to EEPROM write complete

- Priority: Medium

- Consequence of miss: Settings may be lost on power failure

Periodic Tasks:

- PIR sensor polling: Every 100ms

- LED update: Every 50ms (for smooth blinking)

- Timeout counter decrement: Every 1 second

- Watchdog timer reset: Every 2 seconds3.5 Safety and Regulatory Requirements

Electrical Safety Standards:

1. NOM-001-SEDE-2012 (México):

- Compliance with Section 210: Branch circuits

- Section 410: Luminaires and lighting installations

- Section 680: Electrical equipment in hazardous locations (if applicable)

2. IEC 61010-1:

- Safety requirements for electrical equipment for measurement, control, and laboratory use

- Insulation requirements

- Clearance and creepage distances

Safety Features:

SF-01: Electrical Isolation

- Optical isolation between low-voltage control (MCU) and high-voltage power (AC load)

- Minimum creepage distance: 6mm between AC and DC circuits on PCB

SF-02: Overcurrent Protection

- Inline fuse on AC input: 5A fast-blow

- Relay rated for 10A but load limited to 5A maximum via fuse

SF-03: Ground Connection

- Safety ground connected to metal enclosure

- Ground continuity check required during installation

SF-04: Enclosure Rating

- IP20 minimum (protection against solid objects > 12.5mm)

- Non-conductive plastic enclosure to prevent electric shock

SF-05: Thermal Protection

- Components rated for maximum ambient + self-heating

- Relay must not exceed 70°C case temperature at max load

SF-06: Fail-Safe Operation

- On MCU crash/hang: Watchdog timer resets system

- On sensor failure: Light remains in last commanded state (ON or OFF)

- On relay failure: Manual switch bypass remains functional

Regulatory Compliance:

- CE marking (if exported to EU)

- FCC Part 15 Class B (EMI emissions - if applicable)

- RoHS compliance (lead-free components)

- Product safety certification from recognized lab (e.g., UL, CSA, TÜV)

User Safety Warnings (in Manual):

- "Installation must be performed by qualified electrician"

- "Disconnect power before servicing"

- "Do not exceed 500W load rating"

- "Indoor use only - not weatherproof"

- "Do not use in wet or damp locations"